# Battery Pack Requirement Sheet

Template with example values

Template with example values. Replace them with your own and send this sheet with your drawing.

## Battery Pack Requirement Sheet

| Item | Requirement |
| --- | --- |
| Warranty / reliability | 2 years |
| Estimated annual volume | 20,000 units |
| Sales region | China, domestic |
| Sample date / quantity | 5 sets |
| Pilot date / quantity | not stated |
| Production release date / quantity | not stated |
| End customer type | industry customer (B2B) |
| Sample stage | product upgrade |

## 2 · Application

| Item | Requirement |
| --- | --- |
| Description | Replaceable battery pack installed inside roadside equipment |
| Usage pattern | Removed for recharging about once every six months |
| Trickle source | Small solar panel, 0.02C |
| Startup from battery | Small-load start and large-load start both noted |
| Operation mode | Built-in backup power; remains installed while equipment draws current; removable for charging |
| Enclosure | Reuse of existing enclosure tooling preferred |
| Cell strategy | Lowest-cost cell combination to reach the required capacity |

## 3 · Cells and configuration

| Item | Requirement |
| --- | --- |
| Chemistry | NCM lithium |
| Cell format | cylindrical |
| Configuration | 1S xP |
| Minimum capacity | 30 Ah |
| Maximum size (L x W x H) | no requirement yet |
| Weight limit | not stated |
| Life requirement | not stated |

## 4 · Electrical

| Item | Requirement |
| --- | --- |
| Charging method | managed by the host equipment |
| Maximum charge voltage | 4.2 V |
| Charge current | 3.5 A |
| Charge time to full | per cell design |
| Operating voltage range | 3.1 V to 4.2 V |
| Maximum instantaneous discharge current | not stated |
| Maximum continuous discharge current | 5 A |
| Average continuous discharge current | not stated |
| Charging temperature range | 0 to 45 °C, plus solar trickle charging outside this range at 0.02C |
| Discharging temperature range | -20 to 60 °C |
| Storage temperature range | not stated |

## 5 · Protection and monitoring

| Item | Requirement |
| --- | --- |
| Protection functions | over-charge, over-discharge, over-current, short circuit |
| Cell balancing | requested; note on the form asks to confirm whether it is needed given the high parallel count, and its cost |
| Secondary protection | required |
| Fuel gauge | required |
| Communication | none selected (CAN, I2C and SMBUS all unchecked) |

## 6 · Interfaces

| Item | Requirement |
| --- | --- |
| Output | flying lead |
| Connector | 2.0 mm pitch, 6 pins |
| Pin assignment | 1-2 VBAT, 3-4 GND, 5-6 NTC |
| Cable length | not decided |

## 7 · Mechanical and environment

| Item | Requirement |
| --- | --- |
| Installation | replaceable |
| Installation location | outdoors, roadside |
| Ingress protection | IP65 |
| Corrosion | protection board and metal contacts to be waterproof and corrosion resistant |
| ESD / EMC | not stated |
| Customer-side tests | not stated |

## 8 · Certification

| Item | Requirement |
| --- | --- |
| Whole-device certification | not stated |
| Material safety certification | not stated |
| Material environmental certification | not stated |
| Transport certification | not stated |

## Derived values

| Item | Value |
| --- | --- |
| Nominal energy | 30 Ah x 3.6 V = 108 Wh |
| Continuous discharge rate | 5 A / 30 Ah = 0.17C |
| Solar trickle current | 0.02C x 30 Ah = 0.6 A |
| Standby duration | 6 months = 4,380 h |

## Open items to confirm

Life requirement: calendar life, cycle count, and retained capacity at end of warranty

Maximum instantaneous discharge current and duration

Size envelope in mm, or the internal dimensions of the existing enclosure tooling

Operation mode: which of the three stated modes applies

Fuel gauge output: add bus lanes, or voltage indication only

Charging behavior below 0 °C: cutoff temperature and behavior at -10 °C

Certification list, including transport